Final draft

of the

Technical specification for interoperability relating to the subsystem ‘rolling stock — noise’ (NOI TSI)

Amendments record

| Date | Revision | Modifications |
| --- | --- | --- |
| September 2017 | 0.1 | First draft for the WP. - Basis is the existing TSI NOI in force. - Correction of known errors - Alignment with new Directive and modules. - Tracked changes are deviations from the current TSI NOI as voted in RISC. - Parts marked yellow may be elaborated further or disappear, pending the WP opinion. |
| 20 October 2017 | 0.2 | Amended draft during and after kick off meeting - Definition in 2 e improved (during the kick off) and transferred to point 7.2.2.2. - Point 7.2.2.1 created during the meeting. (Wording improved after) - Section 7.3 revised (clarification on specific cases included) - Appendix D, E revised during and after the meeting |
| 3 November 2017 | 0.2.1 | All comments received included. Comments received from: - CER - UNIFE - EIM - NSA FI - NSA NL - NSA PL - NSA SE - OTIF Secretariat |
| 14 November 2017 | 0.3 | Draft to be commented by WP members until 1st December 2017 All comments received in previous draft considered and replied Enacting part added Further changes agreed during WP2 added Enacting part added for further modification |
| 8 December 2017 | 1.0 | Draft to be commented by WP members until 18th January 2018 All comments received in previous draft considered and replied Enacting part added First version of Specific cases added. Guidance for specific cases deleted. Further changes during WP3 Added: - Contents of Appendix N of UIC 541-4 transferred to Appendix F of this TSI. - Appendix D reworded - Appendix E reworded - Appendix F updated with CBBs from Appendix N of UIC 541-4 and additional CBBs notified by NSAs and UIC. |
| 26 January 2018 | 2.0 | Draft to be commented by WP members until 20th February 2018 All comments received in previous draft considered and replied Enacting part removed; relevant clauses for the enacting part identified in the TSI. Second version of Specific cases added. Further changes during WP4 Added: - Deadline added in point 7.2.2 - Appendix D fine-tuned - Appendix E deleted. Contents transferred to point 7.2.2 - Appendix E (former appendix F) updated: - IT blocks included. - UK blocks removed as they are already considered in UK Specific case. Relationship between Appendix N of UIC 541-4 and appendix E clarified. - Parts marked yellow may be elaborated further or disappear, pending the WP opinion |
| 09 March 2018 | Final | Draft to be presented to WP members in WP 5 for last editorial comments/ All comments received in previous draft considered and replied. - 7.2.2 fine-tuned considering hogh gradient lines - Appendix D fine tuned in order to request a list of quieter routes (maps voluntary). - Specific cases updated |

Table of Contents

1.	INTRODUCTION	4

1.1.	Technical scope	4

1.2.	Geographical scope	4

2.	DEFINITION OF THE SUBSYSTEM	4

3.	ESSENTIAL REQUIREMENTS	5

4.	CHARACTERISATION OF THE SUBSYSTEM	5

4.1.	Introduction	5

4.2.	Functional and technical specifications of the subsystems	5

4.2.1.	Limits for stationary noise	6

4.2.2.	Limits for starting noise	7

4.2.3.	Limits for pass-by noise	7

4.2.4.	Limits for the driver´s cab interior noise	8

4.3.	Functional and technical specifications of the interfaces	9

4.4.	Operating rules	9

4.5.	Maintenance rules	10

4.6.	Professional qualifications	10

4.7.	Health and safety conditions	10

4.8.	European register of authorised types of vehicles	10

5.	INTEROPERABILITY CONSTITUENTS	10

6.	CONFORMITY ASSESSMENT AND EC VERIFICATION	11

6.1.	Interoperability constituents	11

6.2.	Subsystem rolling stock regarding noise emitted by rolling stock	11

6.2.1.	Modules	11

6.2.2.	EC verification procedures	11

6.2.3.	Simplified evaluation	14

7.	IMPLEMENTATION	15

7.1.	Application of this TSI to new subsystems	15

7.2.	Application of this TSI to existing subsystems	15

7.3.	Specific cases	17

7.3.1.	Introduction	17

7.3.2.	List of specific cases	17

# INTRODUCTION

In general Technical Specifications for Interoperability (TSI) lay down for each subsystem (or part of it) the optimal level of harmonised specifications in order to ensure the interoperability of the rail system. Therefore TSIs harmonise only the specifications concerning parameters which are critical to interoperability (basic parameters). The specifications of the TSIs must meet the essential requirements as set out in Annex III of Directive 2008/57/EC.

In line with the proportionality principle this TSI sets out the optimal level of harmonisation related to specifications on the rolling stock subsystem as defined in section 1.1 intended to limit the noise emission of the rail system within the Union.

## Technical scope

    - Scope related to rolling stock

This TSI applies to all rolling stock within the scope of Regulation (EU)  No 1302/2014 (LOC&PAS TSI) and Regulation (EU) No 321/2013 (WAG TSI);

    - Scope related to operational aspects

This TSI applies to the operation of freight wagons which are used on infrastructure designated as ‘quieter routes’.

## Geographical scope

The geographical scope of this TSI corresponds to the scopes defined in section 1.2 of LOC&PAS TSI and in section 1.2 of WAG TSI, each for their rolling stock (RST) concerned.

# DEFINITION OF THE SUBSYSTEM

A ‘unit’ means the rolling stock which is subject to the application of this TSI, and therefore subject to the ‘EC’ verification procedure. Chapter 2 of LOC&PAS TSI and chapter 2 of WAG TSI describe what a unit can consist of.

The requirements of this TSI apply to the following categories of rolling stock set out in section 1.2 in Annex I of Directive 2008/57/EC:

- Self-propelling thermal or electric trains. This category is further defined in chapter 2 of LOC&PAS TSI and shall be referred to in this TSI as multiple units, EMU (electrified) or DMU (diesel).

- Thermal or electric traction units. This category is further defined in chapter 2 of LOC&PAS TSI and shall be referred to in this TSI as locomotives. Power units that form part of a “self-propelling thermal or electric train” and railcars are not included in this category and belong to the category under point a).

- Passenger carriages and other related cars. This category is further defined in chapter 2 of LOC&PAS TSI and shall be referred to in this TSI as coaches.

- Freight wagons, including vehicles designed to carry lorries. This category is further defined in chapter 2 of WAG TSI and shall be referred to in this TSI as wagons.

- Mobile railway infrastructure construction and maintenance equipment. This category is further defined in chapter 2 of LOC&PAS TSI and consists of on-track machines (referred to in this TSI as OTMs) and Infrastructure Inspection Vehicles, which belong to the categories in points a), b) or d) depending on their design.

# ESSENTIAL REQUIREMENTS

All basic parameters set out in this TSI must be linked with at least one of the essential requirements as set out in Annex III of Directive 2008/57/EC. Table 1 indicates the allocation.

Table 1:	Basic parameters and their link to the essential requirements

| Point | Basic parameter | Essential requirements |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Safety | Reliability availability | Health | Environm. Protection | Technical compat. |
| 4.2.1 | Limits for stationary noise |  |  |  | 1.4.4 |  |
| 4.2.2 | Limits for starting noise |  |  |  | 1.4.4 |  |
| 4.2.3 | Limits for pass-by noise |  |  |  | 1.4.4 |  |
| 4.2.4 | Limits for driver´s cab interior noise |  |  |  | 1.4.4 |  |

# CHARACTERISATION OF THE SUBSYSTEM

## Introduction

This Chapter sets out the optimal level of harmonisation related to specifications on the rolling stock subsystem intended to limit the noise emission of the Union rail system and to achieve interoperability.

## Functional and technical specifications of the subsystems

The following parameters have been identified as critical for the interoperability (basic parameters)

(a) “stationary noise”,

(b) “starting noise”,

(c) “pass-by noise”,

(d) “driver’s cab interior noise”.

The corresponding functional and technical specifications allocated to the different categories of rolling stock are set out in this section. In case of units equipped with both thermal and electric power the relevant limit values under all normal operation modes shall be respected. If one of these operation modes foresees the use of both thermal and electric power at the same time the less restrictive limit value applies. In accordance with Articles 5(5) and 2(l) of Directive 2008/57/EC, provision may be made for specific cases. Such provisions are indicated in section 7.3.

The assessment procedures for the requirements in this section are defined in the indicated points and sub points of chapter 6.

### Limits for stationary noise

The limit values for the following sound pressure levels under normal vehicle conditions concerning the stationary noise allocated to the categories of the rolling stock subsystem are set out in table 2:

(a)	the A-weighted equivalent continuous sound pressure level of the unit (LpAeq,T[unit]),

(b)	the A-weighted equivalent continuous sound pressure level at the nearest measuring position i considering the main air compressor (LipAeq,T), and

(c)	the AF-weighted sound pressure level at the nearest measuring position i considering impulsive noise of the exhaust valve of the air dryer (LipAFmax).

The limit values are defined at a distance of 7,5 m from the centre of the track and 1,2 m above top of rail.

Table 2:	Limit values for stationary noise

| Category of the rolling stock subsystem | LpAeq,T [unit] [dB] | LipAeq,T [dB] | LipAFmax [dB] |
| --- | --- | --- | --- |
| Electric locomotives and OTMs with electric traction | 70 | 75 | 85 |
| Diesel locomotives and OTMs with diesel traction | 71 | 78 |  |
| EMUs | 65 | 68 |  |
| DMUs | 72 | 76 |  |
| Coaches | 64 | 68 |  |
| Wagons | 65 | n.a. | n.a. |

The demonstration of conformity is described in point 6.2.2.1.

### Limits for starting noise

The limit values for the AF-weighted maximum sound pressure level (LpAF,max) concerning the starting noise allocated to the categories of the rolling stock subsystem are set out in table 3. The limit values are defined at a distance of 7,5 m from the centre of the track and 1,2 m above top of rail.

Table 3:	Limit values for starting noise

| Category of the rolling stock subsystem | LpAF,max [dB] |
| --- | --- |
| Electric locomotives with total tractive power P < 4500 kW | 81 |
| Electric locomotives with total tractive power P ≥ 4500 kW OTMs with electric traction | 84 |
| Diesel locomotives P < 2000 kW at the engine output shaft | 85 |
| Diesel locomotives P ≥ 2000 kW at the engine output shaft OTMs with diesel traction | 87 |
| EMUs with a maximum speed vmax <250 km/h | 80 |
| EMUs with a maximum speed vmax ≥250 km/h | 83 |
| DMUs P < 560 kW/engine at the engine output shaft | 82 |
| DMUs P ≥ 560 kW/engine at the engine output shaft | 83 |

The demonstration of conformity is described in point 6.2.2.2.

### Limits for pass-by noise

The limit values for the A-weighted equivalent continuous sound pressure level at a speed of 80 km/h (LpAeq,Tp,(80 km/h)) and, if applicable, at 250 km/h (LpAeq,Tp,(250 km/h)) concerning the pass-by noise allocated to the categories of the rolling stock subsystem are set out in table 4. The limit values are defined at a distance of 7,5 m from the centre of the track and 1,2 m above top of rail.

Measurements at speeds higher than or equal to 250 km/h shall also be made at the ‘additional measurement position’ with a height of 3,5 m above top of rail in accordance with chapter 6 of EN ISO 3095:2013 and assessed against the applicable limit values of table 4.

Table 4:	Limit values for pass-by noise

| Category of the rolling stock subsystem | LpAeq,Tp (80 km/h) [dB] | LpAeq,Tp (250 km/h) [dB] |
| --- | --- | --- |
| Electric locomotives and OTMs with electric traction | 84 | 99 |
| Diesel locomotives and OTMs with diesel traction | 85 | n.a. |
| EMUs | 80 | 95 |
| DMUs | 81 | 96 |
| Coaches | 79 | n.a. |
| Wagons (normalised to APL=0,225)* | 83 | n.a. |
| *APL: the number of axles divided by the length over the buffers [m-1] |  |  |

The demonstration of conformity is described in point 6.2.2.3.

### Limits for the driver´s cab interior noise

The limit values for the A-weighted equivalent continuous sound pressure level (LpAeq,T) concerning the noise within the driver's cab of electric and diesel locomotives, OTMs, EMUs, DMUs and coaches fitted with a cab are set out in table 5. The limit values are defined in the vicinity of the driver´s ear.

Table 5:	Limit values for driver´s cab interior noise

| Noise within the driver's cab | LpAeq,T [dB] |
| --- | --- |
| At standstill with horns sounding | 95 |
| At maximum speed vmax if vmax < 250 km/h | 78 |
| At maximum speed vmax if 250 km/h ≤ vmax < 350 km/h | 80 |

The demonstration of conformity is described in point 6.2.2.4.

## Functional and technical specifications of the interfaces

This TSI has the following interfaces with the rolling stock subsystem:

Interface with subsystems of points (a), (b), (c) and (e) of chapter 2 (dealt with in LOC&PAS TSI) with regard to

- stationary noise,

- starting noise (not applicable to coaches),

- pass-by noise,

- interior noise within the driver’s cab, where applicable.

Interface with subsystems of point (d) of chapter 2 (dealt with inWAG TSI) with regard to

- pass-by noise,

- stationary noise.

This TSI has the following interface with operation and traffic management subsystem dealt with in Decision 2012/757/EU (OPE TSI) with regard to:

- pass-by noise.

## Operating rules

Requirements concerning the operating rules for the subsystem rolling stock are set out in section 4.4 of LOC&PAS TSI and in section 4.4 of WAG TSI.

4.4.1 Specific rules for the operation of wagons on quieter routes in case of degraded operation

The contingency arrangements as defined in point 4.2.3.6.3 of the OPE TSI include the operation of wagons not compliant with point 7.2.2.2 on quieter routes.

This measure can be applied to address capacity restrictions or operational constraints caused by rolling stock failures, extreme weather conditions and infrastructure failures.

4.4.2 Specific rules for the operation of wagons on quieter routes in case of infrastructure and wagons maintenance

The operation of wagons not compliant with point 7.2.2.2 on quieter routes is possible in case of wagons maintenance activities where only a quieter route is available in order to access the maintenance workshop.

Contingency arrangements set out in clause 4.4.1 are applicable in case of infrastructure maintenance, renewal or upgrade activities where a quieter route is the only suitable alternative.

## Maintenance rules

Requirements concerning the maintenance rules for the subsystem rolling stock are set out in section 4.5 of LOC&PAS TSI and in section 4.5 of WAG TSI.

## Professional qualifications

Not applicable.

## Health and safety conditions

See Article 6 of this Regulation.

## European register of authorised types of vehicles

The data of the rolling stock that must be recorded in the “European register of authorised types of vehicles (ERATV)” are set out in Decision 2011/665/EU.

# INTEROPERABILITY CONSTITUENTS

There is no interoperability constituent specified in this TSI.

# CONFORMITY ASSESSMENT AND EC VERIFICATION

## Interoperability constituents

Not applicable.

## Subsystem rolling stock regarding noise emitted by rolling stock

### Modules

The EC verification shall be performed in accordance with the module(s) described in table 6.

Table 6:	Modules for EC verification of subsystems

| SB | EC-Type Examination |
| --- | --- |
| SD | EC verification based on quality management system of the production process |
| SF | EC verification based on product verification |
| SH1 | EC verification based on full quality management system plus design examination |

These modules are specified in detail in Decision 2010/713/EU.

### EC verification procedures

The applicant shall choose one of the following assessment procedures consisting of one or more modules for the EC verification of the subsystem:

- (SB+SD),

- (SB+SF),

- (SH1).

Within the application of the chosen module or module combination the subsystem shall be assessed against the requirements defined in section 4.2. If necessary, additional requirements concerning the assessment are given in the following points.

#### Stationary noise

The demonstration of conformity with the limit values on stationary noise as set out in point 4.2.1 shall be carried out in accordance with sections  5.1, 5.2, 5.3, 5.4, 5.5 (without clause 5.5.2), 5.7 and clause 5.8.1 of EN ISO 3095:2013.

For the assessment of the main air compressor noise at the nearest measuring position i, the LipAeq,T indicator shall be used with T representative of one operating cycle as defined in section 5.7 of EN ISO 3095:2013. Only the train systems that are required for the air compressor to run under normal operating conditions shall be used for this. The train systems which are not needed for the operation of the compressor may be switched off to prevent contribution to the noise measurement. The demonstration of conformity with the limit values shall be carried out under the conditions solely necessary for operation of the main air compressor at the lowest rpm.

For the assessment of the impulsive noise sources at the nearest measuring position i, the LipAFmax indicator shall be used. The relevant noise source is the exhaust from the valves of the air dryer.

#### Starting noise

The demonstration of conformity with the limit values on starting noise as set out in point 4.2.2 shall be carried out in accordance with chapter 7 (without clause 7.5.1.2) of EN ISO 3095:2013. The maximum level method referring to section 7.5 of EN ISO 3095:2013 shall apply. Deviating from clause 7.5.3 of EN ISO 3095:2013 the train shall accelerate from standstill up to 30 km/h and then maintain the speed.

In addition the noise shall be measured at a distance of 7,5m from the centre of the track and a height of 1,2 m above top of rail. The “averaged level method” and the “maximum level method” in accordance with section 7.6 and 7.5 respectively of EN ISO 3095:2013 shall apply and the train shall accelerate from standstill up to 40 km/h and then maintain the speed. The measured values are not assessed against any limit value and shall be recorded in the technical file and communicated to the Agency.

For OTMs the starting procedure shall be performed without additional trailer loads.

#### Pass-by noise

The demonstration of conformity with the limit values on pass-by noise as set out in point 4.2.3 shall be carried out in accordance with points 6.2.2.3.1 and 6.2.2.3.2.

6.2.2.3.1 Test track conditions

The tests shall be performed on a reference track as defined in section 6.2 of EN ISO 3095:2013.

However, it is permitted to carry out the test on a track that does not comply with the reference track conditions in terms of acoustic rail roughness level and track decay rates as long as the noise levels measured in accordance with point 6.2.2.3.2 do not exceed the limit values set out in point 4.2.3.

The acoustic rail roughness and the decay rates of the test track shall be determined in any case. If the track on which the tests are performed does meet the reference track conditions, the measured noise levels shall be marked ‘comparable’, otherwise they shall be marked ‘non-comparable’. It shall be recorded in the technical file whether the measured noise levels are ‘comparable’ or ‘non-comparable’.

The measured acoustic rail roughness values of the test track remain valid during a period starting 3 months before and ending 3 months after this measurement, provided that during this period no track maintenance has been performed which influences the rail acoustic roughness.

The measured track decay rate values of the test track shall remain valid during a period starting 1 year before and ending 1 year after this measurement, provided that during this period no track maintenance has been performed which influences the track decay rates.

Confirmation shall be provided in the technical file that the track data related to the type’s pass-by noise measurement were valid during the day(s) of testing, e.g. by providing the date of last maintenance having an impact on noise.

Furthermore, it is permitted to carry out tests at speeds equal to or higher than 250 km/h on slab tracks. In this case the limit values shall be 2 dB higher than those set out in point 4.2.3.

6.2.2.3.2 Procedure

The tests shall be carried out in accordance with the provision in sections 6.1, 6.3, 6.4, 6.5, 6.6 and 6.7 (without 6.7.2) of EN ISO 3095:2013. Any comparison against limit values shall be carried out with results rounded to the nearest integer decibel. Any normalisation shall be performed before rounding.  The detailed assessment procedure is set out in points 6.2.2.3.2.1, 6.2.2.3.2.2 and 6.2.2.3.2.3.

6.2.2.3.2.1 EMU, DMUs, locomotives and coaches

For EMU, DMUs, locomotives and coaches three classes of maximum operational speed are distinguished:

- If the maximum operational speed of the unit is lower than or equal to 80 km/h, the pass-by noise shall be measured at its maximum speed vmax. This value shall not exceed the limit value LpAeq,Tp(80 km/h) as set out in point 4.2.3.

- If the maximum operational speed vmax of the unit is higher than 80 km/h and lower than 250 km/h, the pass-by noise shall be measured at 80 km/h and at its maximum speed. Both measured pass-by noise values LpAeq,Tp(vtest) shall be normalised to the reference speed of 80 km/h LpAeq,Tp(80 km/h) using formula (1). The normalised value shall not exceed the limit value LpAeq,Tp(80 km/h) as set out in point 4.2.3.

LpAeq,Tp(80 km/h) = LpAeq,Tp(vtest) - 30*log (vtest/80 km/h)					(1)

vtest = Actual speed during the measurement

- If the maximum operational speed vmax of the unit is equal to or higher than 250 km/h, the pass-by noise shall be measured at 80 km/h and at its maximum speed with an upper test speed limit of 320 km/h. The measured pass-by noise value LpAeq,Tp(vtest) at 80 km/h shall be normalised to the reference speed of 80 km/h LpAeq,Tp(80 km/h) using formula (1). The normalised value shall not exceed the limit value LpAeq,Tp(80 km/h) as set out in point 4.2.3. The measured pass-by noise value at maximum speed LpAeq,Tp(vtest) shall be normalised to the reference speed of 250 km/h LpAeq,Tp(250 km/h)  using formula (2). The normalised value shall not exceed the limit value LpAeq,Tp(250 km/h) as set out in point 4.2.3.

LpAeq,Tp(250 km/h) = LpAeq,Tp(vtest) - 50*log(vtest/250 km/h)					(2)

vtest = Actual speed during the measurement

6.2.2.3.2.2 Wagons

For wagons two classes of maximum operational speed are distinguished:

- If the maximum operational speed vmax of the unit is lower than or equal to 80 km/h, the pass-by noise shall be measured at its maximum speed. The measured pass-by noise value LpAeq,Tp(vtest) shall be normalised to a reference APL of 0,225 m-1 LpAeq,Tp (APLref) using formula (3). This value shall not exceed the limit value LpAeq,Tp(80 km/h) as set out in point 4.2.3.

LpAeq,Tp (APLref) = LpAeq,Tp(vtest) - 10*log(APLwag/0,225 m-1)					(3)

APLwag = Number of axles divided by the length over the buffers [m-1]
			vtest = Actual speed during the measurement

- If the maximum operational speed vmax of the unit is higher than 80 km/h, the pass-by noise shall be measured at 80 km/h and at its maximum speed. Both measured pass-by noise values LpAeq,Tp(vtest) shall be normalised to the reference speed of 80 km/h and to a reference APL of 0,225 m-1 LpAeq,Tp(APL ref, 80 km/h) using formula (4). The normalised value shall not exceed the limit value LpAeq,Tp(80 km/h) as set out in point 4.2.3.

LpAeq,Tp (APLref, 80 km/h) = LpAeq,Tp(vtest) - 10*log(APLwag/0,225 m-1) – 30*log(vtest/80 km/h)	(4)

APLwag = Number of axles divided by the length over the buffers [m-1]
		vtest     = Actual speed during the measurement

6.2.2.3.2.3 OTMs

For OTMs the same assessment procedure as set out in 6.2.2.3.2.1 applies. The measuring procedure shall be performed without additional trailer loads.

OTMs are deemed to comply with the pass-by noise level requirements in point 4.2.3 without measuring when they are:

- solely braked by either composite brake blocks or disc brakes and

- equipped with composite scrubbers, if scrubber blocks are fitted.

#### Driver´s cab interior noise

The demonstration of conformity with the limit values on the driver´s cab interior noise as set out in point 4.2.4 shall be carried out in accordance with EN 15892:2011. For OTMs the measuring procedure shall be performed without additional trailer loads.

### Simplified evaluation

Instead of the test procedures as set out in point 6.2.2, it is permitted to substitute some or all of the tests by a simplified evaluation. The simplified evaluation consists of acoustically comparing the unit under assessment to an existing type (further referred to as the reference type) with documented noise characteristics.

The simplified evaluation may be used for each of the applicable basic parameters “stationary noise”, “starting noise”, “pass-by noise” and “driver´s cab interior noise” autonomously and shall consist of providing evidence that the effects of the differences of the unit under assessment do not result in exceeding the limit values set out in section 4.2.

For the units under simplified evaluation, the proof of conformity shall include a detailed description of the noise relevant changes compared to the reference type. From this description, a simplified evaluation shall be performed. The estimated noise values shall include the uncertainties of the applied evaluation method. The simplified evaluation can either be a calculation and/or simplified measurement.

A unit certified on the basis of the simplified evaluation method shall not be used as a reference unit for a further evaluation.

If the simplified evaluation is applied for pass-by noise, the reference-type shall comply with at least one of the following:

- Chapter 4 and for which the pass-by noise results are marked ‘comparable’

- Chapter 4 of Decision 2011/229/EU and for which the pass-by noise results are marked ‘comparable’

- Chapter 4 of Decision 2006/66/EC

- Chapter 4 of Decision 2008/232/EC.

In case of a wagon which parameters remain, compared to the reference type, within the permitted range of table 7 it is deemed without further verification that the unit complies with the limit values on pass-by noise as set out in point 4.2.3.

Table 7:	Permitted variation of wagons for the exemption from verification

| Parameter | permitted variation (compared to the reference unit) |
| --- | --- |
| Max. unit Speed | Any speed up to 160 km/h |
| Type of wheel | Only if equally or less noisy (acoustic characterisation i. a. w. Annex E of EN 13979-1:2011) |
| Tare weight | Only within the range of +20% / - 5% |
| Brake block | Only if variation does not result in higher noise emission. |

# IMPLEMENTATION

## Application of this TSI to new subsystems

See Article 8 of this Regulation.

## Application of this TSI to existing subsystems

7.2.1 General provisions in case of renewal or upgrade

The applicant shall demonstrate that the noise levels of renewed or upgraded units remain below the limits set out in the TSI which was applicable when the unit in question was first authorised. If no TSI existed at the time of the first authorisation, it shall be demonstrated that the noise levels of renewed or upgraded units are either not increased or remain below the limits set out in Decision 2006/66/EC or Decision 2002/735/EC.

The demonstration shall be limited to the basic parameters affected by the renewal/upgrade.

If the simplified evaluation is applied, the original unit may represent the reference unit in accordance with the provisions of point 6.2.3.

The replacement of a whole unit or (a) vehicle(s) within a unit (e.g. a replacement after a severe damage) does not require a conformity assessment against this TSI, as long as the unit or the vehicle(s) are identical to the ones they replace.

7.2.2 Additional provisions for the application of this TSI to existing wagons

From 8th  December 2024 wagons which do not fulfil the conditions set out in point 7.2.2.2 of this TSI shall not be operated on the quieter routes as defined in Appendix D.

The provision above does not apply to wagons which are operated on lines with a gradient of more than 40‰, wagons exclusively operated for infrastructure maintenance and construction works and to wagons used in rescue trains.

If a wagon is being equipped with quieter brake blocks as defined in 7.2.2.1 and no noise sources are added to the wagon, then it shall be assumed that the requirements of point 4.2.3 are met without further testing.

7.2.2.1 Quieter brake blocks

A quieter brake block is a brake block belonging to one of the following categories:

- Brake block listed in Appendix G of WAG TSI

- Brake block assessed in accordance with the procedure set out in Appendix F of this TSI.

7.2.2.2 Wagons operated on quieter routes

Wagons belonging to one of the categories below can be operated on quieter routes:

  - Wagons holding an EC declaration of verification against Commission Decision 2006/66/EC concerning the technical specification for interoperability relating to the subsystem ‘rolling stock — noise’ of the trans-European conventional rail system.

  - Wagons holding an EC declaration of verification against Commission Decision 2011/229/EU concerning the technical specifications of interoperability relating to the subsystem ‘rolling stock – noise’ of the trans-European conventional rail system.

  - Wagons holding an EC declaration of verification against this TSI.

  - Wagons fitted with quieter brake blocks as defined in 7.2.2.1 or brake discs for the service brake function.

  - Wagons fitted with composite brake blocks listed in appendix E. The operation of these wagons on quieter routes is limited in accordance with the conditions described in this appendix.

## Specific cases

### Introduction

The specific cases, as listed in point 7.3.2, are classified as

- “P” cases: “permanent” cases.

- “T” cases: “temporary” cases.

### List of specific cases

#### General specific case

a) Specific case Estonia, Finland, Latvia, Lithuania and Poland

(“P”) For units which are in shared use with third countries, the track gauge of which is different from that of the main rail network within the Union, the application of national technical rules instead of the requirements in this TSI is permitted.

b) Specific case Finland

(“T”) Decision 2011/229/EU may continue to be applied for freight wagons to be used only on the territory of Finland and until the relevant technical solution in relation to Nordic winter conditions is found, but in any case not later than until 31/12/2032. This shall not prevent freight wagons from other Member States to operate on the Finnish network. Thus, the concept of  quieter routes is not applicable for Finnish 1524 mm network.

#### Limits for stationary noise (point 4.2.1)

a) Specific case Finland

(“T”) For coaches and wagons equipped with a diesel generator for electrical power supply higher than 100 kW and intended to operate solely on the railway network of Finland the limit value for stationary noise LpAeq,T [unit] in table 2 may be raised up to 72 dB.

b) Specific case UK for Great Britain

(“P”) For DMUs intended to operate solely on the railway network of Great Britain the limit value for stationary noise LpAeq,T [unit] in table 2 may be raised up to 77 dB.

This specific case does not apply to DMUs intended to operate solely on the High Speed 1 railway network.

c) Specific case UK for Great Britain

(“T”) For units intended to operate solely on the railway network of Great Britain the limit values LipAeq,T in table 2 considering the main air compressor do not apply.  The measured values shall be submitted to the NSA UK.

This specific case does not apply to units intended to operate solely on the High Speed 1 railway network.

#### Limits for starting noise (point 4.2.2)

a) Specific case Sweden

(“T”) For locomotives with total tractive power of more than 6 000 kW and a maximum axle load of more than
25 t the limit values for starting noise LpAF,max in table 3 may be raised up to 89 dB.

b) Specific case UK for Great Britain

(“P”) For units specified in table 8 intended to operate solely on the railway network of Great Britain the limit value for starting noise LpAF,max in table 3 may be raised up to the values set out in table 8.

Table 8:	Limit values for starting noise regarding a specific case UK for Great Britain

| Category of the rolling stock subsystem | LpAF,max [dB] |
| --- | --- |
| Electric locomotives with total tractive power P < 4500 kW | 83 |
| Diesel locomotives P < 2000 kW at the engine output shaft | 89 |
| DMUs | 85 |

This specific case does not apply to units intended to operate solely on the High Speed 1 railway network.

#### Limits for pass-by noise (point 4.2.3)

a) Specific case Sweden

(“T”) For locomotives with total tractive power of more than 6 000 kW and a maximum axle load of more than 25 t the limit values for pass-by noise LpAeq,Tp (80 km/h) in table 4 may be raised up to 85 dB.

#### Additional provisions for the application of this TSI to existing wagons (point 7.2.2)

a) Specific case Finland

(“T”) Concept of quieter routes will not be applied on Finnish network due to uncertainties related to the operation in severe winter conditions with composite brake blocks until 31/12/2032.

#### Wagons operated on quieter routes (point 7.2.2.2)

a) Specific case Belgium

(“T”) On top of the wagons listed in point 7.2.2.2, the following existing wagons can be operated on quieter routes in the territory of Belgium:

- Wagons with tyred wheels

- Wagons which require the fitting of a kink valve in order to replace the cast iron block with composite brake blocks

- Wagons fitted with cast iron blocks which require the replacement of wheels with wheels compliant with the requirements set out in EN 13749-1 in order to be retrofitted with composite brake blocks

b) Specific case Czech Republic

(“T”) On top of the wagons listed in point 7.2.2.2, the following existing wagons can be operated on quieter routes in the territory of Czech Republic:

- Wagons with tyred wheels, until 1st January 2027

- Wagons with 1Bg or 1Bgu brake configuration fitted with cast iron blocks

- Wagons which require the fitting of a kink valve in order to replace the cast iron block with composite brake blocks

c) Specific case France

(“T”) On top of the wagons listed in point 7.2.2.2, the following existing wagons can be operated on quieter routes in the territory of France:

- Wagons with 1Bg or 1Bgu brake configuration fitted with cast iron brake blocks

- Wagons fitted with small wheels (diameter under 920mm)

d) Specific case Italy

(“T”) On top of the wagons listed in point 7.2.2.2, the following existing wagons can be operated on quieter routes in the territory of Italy:

- Wagons with tyred wheels until 2026;

- Wagons which require the fitting of a kink valve in order to replace the cast iron block with composite brake blocks until 2026;

- Wagons fitted with cast iron blocks which require the replacement of wheels with wheels compliant with the requirements set out in EN 13749-1 in order to be retrofitted with composite brake blocks until 2026;

Furthermore, it is not mandatory to use composite brake blocks on quieter routes for existing wagons not covered by the three bullet points above and for which there exists no 1-to-1-solution for replacement of cast iron brake blocks until 2030.

e) Specific case Norway

(“T”) The use of composite brake blocks on any freight wagon for use on quieter routes within the Norwegian railway network is not mandatory until xx/xx/xxxx.

When wagons equipped with composite brake blocks are used during severe winter conditions the RU shall take necessary provisions through their SMS in order to assure satisfactory braking performance for the train composition..

(“T”) Furthermore, it is not mandatory to use composite brake blocks on quieter routes for wagons for which there exists no 1-to-1-solution for replacement of cast iron brake blocks.’

f) Specific case Poland

(“T”) On top of the wagons listed in point 7.2.2.2, the following existing wagons can be operated on quieter routes in the territory of Poland:

- Wagons with tyred wheels, until 1st January 2037

- Wagons with 1Bg or 1Bgu brake configuration fitted with cast iron blocks

- Wagons designed for “S” traffic equipped with “SS” brake fitted with cast iron blocks

- Wagons fitted with cast iron blocks and designed for “SS” traffic for which retrofitting with LL brake blocks would require fitting with wheels complying with EN 13979-1 and a kink valve.

e) Specific case Sweden

(“T”) The use of composite brake blocks is not mandatory on freight wagons for use on quieter routes within the Swedish railway network.

The railway undertakings must take necessary operative provisions when using wagons equipped with composite brake blocks in severe winter conditions in order to assure satisfactory braking performance for the train composition.

(“P”) It is not mandatory to use composite brake blocks on existing freight wagons for which there exists no 1-to-1-solution for replacement of cast iron brake blocks on quieter routes within the Swedish railway network.

g) Specific case UK for Great Britain

(“P”) For Units intended to operate solely on the GB Network, where wagons are equipped with composite brake blocks published in GMGN 2688 it shall be permissible to operate over quieter routes.

This specific case does not prevent the access of TSI compliant rolling stock to the national network

(“P”) For Units intended to operate solely on the GB Network, where wagons equipped with cast iron historic brake blocks, it shall be permissible to operate over quieter routes where there exists no 1-to-1-solution for replacement of cast iron brake blocks.

This specific case does not prevent the access of TSI compliant rolling stock to the national network

Appendix A - Open points

| Element of the Rolling Stock subsystem | Clause of this TSI | Technical aspect not covered by this TSI | Comments |
| --- | --- | --- | --- |
| Quieter brake block | 7.2.2.1 and Appendix F | Assessment of the acoustic properties of the brake blocks | Alternative technical solutions available (see point 7.2.2) |

Appendix B - Standards referred to in this TSI

| TSI |  | Standard |  |
| --- | --- | --- | --- |
| Characteristics to be assessed |  | References to mandatory standards | Chapter |
| Stationary noise | 4.2.1 | - | - |
|  | 6.2.2.1 | EN ISO 3095:2013 | 5 |
| Starting noise | 4.2.2 | - | - |
|  | 6.2.2.2 | EN ISO 3095:2013 | 7 |
| Pass-by noise | 4.2.3 | EN ISO 3095:2013 | 6 |
|  | 6.2.2.3 | EN ISO 3095:2013 | 6 |
| Driver´s cab interior noise | 4.2.4 | - | - |
|  | 6.2.2.4 | EN 15892:2011 | all |
| Simplified evaluation | 6.2.3 | EN 13979-1:2011 | Annex E |

Appendix C	Assessment of the rolling stock subsystem

| Characteristics to be assessed, as specified in section 4.2 |  |  |  |  | Particular assessment procedure |
| --- | --- | --- | --- | --- | --- |
|  |  | Design review | Type Test | Routine Test |  |
| Element of the Rolling Stock sub-system | Point |  |  |  | Point |
| Stationary noise | 4.2.1 | X* | X | n.a. | 6.2.2.1 |
| Starting noise | 4.2.2 | X* | X | n.a. | 6.2.2.2 |
| Pass-by noise | 4.2.3 | X* | X | n.a. | 6.2.2.3 |
| Driver´s cab interior noise | 4.2.4 | X* | X | n.a. | 6.2.2.4 |

* Only if the simplified evaluation in accordance with point 6.2.3 is applied.

Appendix D	Quieter routes

D.1 Definition

A ‘quieter route’ is a part of the network with a minimum length of 20 km in the geographical scope of this TSI, on which the annual average daily operated freight trains in the year preceeding the publication date of Regulation xx/xxxx [amendment to TSI NOISE] during night time was higher than 12.

Night time is defined for each Member State in its national legislation transposing Directive 2002/49/EC.

D.2 Identification of quieter routes

The Member States shall provide the Agency with a list of quieter routes no later than 3 months after the date of publication of this TSI.

The list shall contain at least the following information:

- Start and end point of the quieter route. If one of these points is at the border of the Member State, it shall be reflected.

- Identification of the line(s) making up the quieter route

- Number of freight trains circulating during night on the quieter route

The Member States may provide maps illustrating the quieter routes on a voluntary basis.All lists and maps shall be published on the Agency website (http://www.era.europa.eu) no later than 6 months after the date of publication of this TSI.

D.3 Update of quieter routes

Member States shall update the quieter routes at least every 5 years after 8th December 2024. The traffic data used shall refer to the year preceding the update. Member States shall provide the Agency with the updated quieter routes for their publication. In case of new or renewed lines added to the existing network, the expected traffic can be used for classification.

The Agency shall inform the Commission of any changes to the quieter routes. The Commission shall inform the Member States of these changes through the committee referred to in Article 51 of Directive (EU) 2016/797. The updates shall be applicable from the next December timetable change following one year after the Commission has informed the Member States of the changes. The Agency shall publish the updated quieter routes on its website (http://www.era.europa.eu) one month after the Commission has informed the Member States.

Appendix E – Historic composite brake blocks

E.1 Historic composite brake blocks for international use.

Existing wagons equipped with the brake blocks listed below are allowed to be used on the EU railway network, including quieter routes, until the relevant date set out in Appendix N of UIC 541-4.

| Manufacturer/name of product | Designation/type of block | Type of friction coefficient |
| --- | --- | --- |
| Valeo/Hersot Wabco/Cobra | 693 W554 | K |
| Wabco/Cobra | W 392 | K |
| Ferodo | I/B 436 | K |
| Abex | 229 | K (Fe - sintered) |
| Mintex DON | TBL 804 | K |
| Jurid | 738 | K (Fe - sintered) |

E.2 Historic composite brake blocks for national use

Existing wagons equipped with the brake blocks listed below are only allowed to be used on the railway networks, including quieter routes, of the corresponding Member States.

| Manufacturer/name of the product | Designation/type of block | Member State |
| --- | --- | --- |
| Becorit | 929-1 | Switzerland |
| Cobra/Wabco | V133 | Italy |
| Cofren | M128 Mix S153 | Norway, Sweden |
| Cofren | 229 | Italy |
| Federal Mogul | J816M | Italy |
| ICER | 904 | Spain |
| ICER | 905 | Spain, Portugal (TBC its use in Portugal) |
| Jurid | 816 | Switzerland |
| Jurid | 838 | Spain |
|  | S 153 | Norway, Sweden |
| Wabtec | 333 | Norway, Sweden |

Appendix F – Assessment of the acoustic performance of a brake block

The purpose of this procedure is to demonstrate the acoustic performance of a  composite brake block at interoperability constituent level.

This procedure is an open point.